BAHRIA UNIVERSITY (KARACHI CAMPUS) MIDTERM ASSESSMENT – SPRING 2020 Digital Design (CEN- 122) Class: BSE-2A & 2B Max Marks: 20 Faculty Member: Engr. Mahawish Due Date: 31st May 2020 Student’s Name: _AHMED FARAZ______ Reg. #: __02-131192-037___ Note: - All questions should be answered in the same file using black font color. - Write the answer in your own wordings and avoid copying/pasting. - Your submitted report might be checked for plagiarism.

# Q: 1 [2.0 Marks]                                                                                                                      [CLO 2]

Encode the expression “my name is ____.” (e.g., my name is Sana) in ASCII using an eight-bit code also include the period and space.

Answer:

Expression:

“my name is Ahmed Faraz”

ASCII:

341091213211097109103321051153265104109991003270971149712234

Code using 8-bit:

0010001001101101011110010010000001101110011000010110110101100101001000000110100101111001100100000010000010110100001101101011001110110010000100000010001100110000101110010011000010111101000100010

# Q: 2 [2.0 Marks]                                                                                                                      [CLO 2]

Perform the subtraction with the following unsigned decimal numbers by taking the 10's complement of the subtrahend.

- 5250 - 1321                                                                                                                               (0.5)

Solution:

Subtrahend = 1321

Taking 9’s complement

9999

-1321

8678

Taking 10’s complement

8678

+1

8679

Now Adding,

5250

+8679

1|3929

Final Answer = 3929

Verification:

5250

-1321

3929

- 1753 - 8640                                                                                                                               (0.5)

Solution:

Subtrahend = 8640

Taking 9’s complement

9999

-8640

1359

Taking 10’s complement

1359

+1

1360

Now Adding,

1753

+1360

3113

Introducing -ve sign by taking 10’s complement of 3113

Taking 9’s complement

9999

-3113

6886

Taking 10’s complement

6886

+1

6887

Final Answer = 6887

Verification:

1753

-8640

-6887

- 20 - 10                                                                                                                                       (0.5)

Solution:

Subtrahend = 10

Taking 9’s complement

99

-10

89

Taking 10’s complement

89

+1

90

Now Adding,

90

+20

1|10

Final Answer = 10

Verification:

20

-10

10

- 1200 – 250                                                                                                                                (0.5)

Solution:

Subtrahend = 250

Taking 9’s complement

999

-250

749

Taking 10’s complement

749

+1

750

Now Adding,

1200

+750

1|950

Final Answer = 950

Verification:

1200

-250

950

# Q: 3 [4.0 Marks]                                                                                                                      [CLO 2]

Determine the SOP and POS expressions for the following function and also draw the circuit diagram.

F (x1, x2, x3, x4) = Σm(1, 3, 5, 7, 9, 11, 15) + d(2, 4, 12, 14)

Answer:

# Q: 4 [8.0 Marks]                                                                                                                     [CLO 2]

Minimize the following Boolean expression using Boolean laws also write the name of Boolean identities in minimize each step.

- A.B (A+B) (1+B’)                                                                                                                   (2.0)

AAB +ABB(1+B’)          Idempotent law

AB+AB (1+B’)               Idempotent law

AB(1+B’)

AB+ABB’                       Inverse law

AB                                  Final Answer

- (1 + C) (A.D + A.D’) + A.C + C                                                                                            (2.0)

(A.D + A.D’) + A.C + C               Null law

A1+A.C+C	                             Inverse law

A+A.C+C

A(1+C)+C                                    Null law

A+C                                              Final Answer

- (A.C) + (B.C’)                                                                                                                         (2.0)

Solution not possible

- ((A.B)+A).((B.B)+B’))                                                                                                            (2.0)

A(B+1) (B+B’)                     Null law

A                                           Inverse law

# Q: 5 [2.0 Marks]                                                                                                                      [CLO 3]

Design a combinational circuit with three inputs and one output. The output is 1 when the binary value of the inputs is less than 3. The output is 0 otherwise.

Truth Table:

| a | b | c | d |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 0 |

K-Map and Circuit Diagram:

# Q6: [2.0 Marks]                                                                                                                       [CLO 3]

Design the output waveform for XNOR, AND, NOR and XOR logic gates the two inputs A and B are given below.

Page 1 of 2

Answer:

Good Luck

Page 2 of 2