| 1 | LED |
| --- | --- |
| 2 | SGD |
| 3 | LCD |
| 4 | Keypad |
| 5 | PIR done |
| 6 | Variable Resistor |
| 7 | RGB LED |
| 8 | Ultrasonic |
| 9 | Soil Moisture |
| 10 | Servo |
| 11 | DC motor |
| 12 | LM35 |
| 13 | LDR |
| 14 |  |
| 15 | 12C |

Perform rotation operation:

Assembly code:

$MOD51

ORG 0000h

MOV A, #001h

start:

MOV P2, A

ACALL delay

RR A

ACALL delay

SJMP start

delay:

MOV R0, #010h

MOV R1, #0ffh

back:

DJNZ R1, back

DJNZ R0, back

ret

END

Perform four on and off

Assembly Code:

$MOD51	; This includes 8051 definitions for the Metalink assembler

; Please insert your code here.

org 0000h;

mov A,#0F5h

mov A,#055h

start:

mov p2,A

acall delay

cpl A

sjmp start

delay:

mov r0,#010h

mov r1,#0ffh

back: djnz r1,back

djnz r0, back

ret

END

Two on and six off:

Assembly Code

$MOD51	; This includes 8051 definitions for the Metalink assembler

; Please insert your code here.

org 0000h;

mov A,#0F5h

mov B,#055h

start:

mov p2,A

acall delay

cpl A

sjmp start

delay:

mov r0,#010h

mov r1,#0ffh

back: djnz r1,back

djnz r0, back

ret

END

C Code:

Perform rotation operation:

#include <8051.h>

void main(){

while(1){

P2 = #0AAH;

Delay();

P2 = #055H;}

}

void Delay(){

for(int i=0;i<255;i++);

}

Perform four on and off

#include <8051.h>

void delay() {

unsigned int i, j;

for(i = 0; i < 500; i++)

for(j = 0; j < 1000; j++);

}

void main() {

unsigned char pattern = 0x0F;

while(1) {

P1 = ~pattern;

P2 = ~pattern;

P3 = ~pattern;

delay();

P1 = 0xFF;

P2 = 0xFF;

P3 = 0xFF;

delay();

pattern = ~pattern; // Toggle pattern (four on to four off)

}

}

Two on and six off:

#include <8051.h>

void delay() {

unsigned int i, j;

for(i = 0; i < 500; i++)

for(j = 0; j < 1000; j++);

}

void main() {

unsigned char pattern = 0x03; // Initial pattern (two LEDs on, six off)

while(1) {

P1 = ~pattern;

P2 = ~pattern;

P3 = ~pattern;

delay();

P1 = 0xFF; // Turn off LEDs on P1

P2 = 0xFF;

P3 = 0xFF;

delay();

}

}

Program of LED through switch

#include<reg51.h>

sbit sw=P1^0;

void main()

{      while(1)

{

if( sw == 0)

{

P2 = 0xFF;

}

else

{

P2 = 0x00;

}

}

}

Seven Segment Display:

- Draw the Block diagram of seven segment display on Multisim/Proteous and write its code in C and Assembly

C-Code:

#include <htc.h>

#define Port_1 P2

void delay(int ms) {

int i, j;

for (i = 0; i < ms; i++) {

for (j = 0; j < 100; j++);

}

}

void display(int a){

switch(a){

case 0:

Port_1=0x40;

break;

case 1:

Port_1=0x79;

break;

case 2:

Port_1=36;

break;

case 3:

Port_1=48;

break;

case 4:

Port_1=0x19;

break;

case 5:

Port_1=0x12;

break;

case 6:

Port_1=0x02;

break;

case 7:

Port_1=0x78;

break;

case 8:

Port_1=0x00;

break;

case 9:

Port_1=0x10;

break;

}

}

Draw block diagram of two seven segment display. Write a program in Assembly and C to display a digit from 0 to 9 in both seven segment display

C Code:

#include <htc.h>

#define Port_0 P0

#define Port_1 P2

void delay(int ms) {

int i, j;

for (i = 0; i < ms; i++) {

for (j = 0; j < 100; j++);

}

}

void display(int a){

switch(a){

case 0:

Port_0=0x40;

Port_1=0x10;

break;

case 1:

Port_0=0x79;

Port_1=0x00;

break;

case 2:

Port_0=36;

Port_1=0x78;

break;

case 3:

Port_0=48;

Port_1=0x02;

break;

case 4:

Port_0=0x19;

Port_1=0x12;

break;

case 5:

Port_0=0x12;

Port_1=0x19;

break;

case 6:

Port_0=0x02;

Port_1=48;

break;

case 7:

Port_0=0x78;

Port_1=36;

break;

case 8:

Port_0=0x00;

Port_1=0x79;

break;

case 9:

Port_0=0x10;

Port_1=0x40;

break;

}

}

LCD

C-Code:

#include <LiquidCrystal.h>

LiquidCrystal lcd(10,9,8,7,6,5);

void setup() {

lcd.begin(16, 2);

}

void loop() {

lcd.setCursor(0, 1);

lcd.print("Wellcome To Bahria");

}

Output:

CIRCUIT:

LCD WITH KEYPAD

Diagram:

Variable Resistor/ Potentiometer

#include <LiquidCrystal.h>

// Initialize the LCD with the specified pins

LiquidCrystal lcd(13, 12, 5, 4, 3, 2);

void setup() {

lcd.begin(16, 2);  // Initialize the LCD with 16 columns and 2 rows

lcd.print("Voltage: ");

}

void loop() {

int sensorValue = analogRead(A0); // Read analog value from the potentiometer

float voltage = sensorValue * (3.75 / 1023.0); // Convert analog value to voltage

lcd.setCursor(9, 0);

lcd.print(voltage, 2); // Display voltage

float error = 3.75 - voltage; // Calculate error

lcd.setCursor(0, 1);

lcd.print("Error: ");

lcd.print(error, 2); // Display error

delay(1000); // Wait for a second

}

OUTPUT:

<LiquidCrystal.h>

#include <Keypad.h>

LiquidCrystal lcd(13, 12, 11, 10, 9, 8);

const byte ROW_NUM    = 4;

const byte COLUMN_NUM = 4;

char keys[ROW_NUM][COLUMN_NUM] = {

{'1','2','3'},

{'4','5','6'},

{'7','8','9'},

{'*','0','#'}

};

byte pin_rows[ROW_NUM] = {4, 3, 2, 1};

byte pin_column[COLUMN_NUM] = {7, 6, 5};

Keypad keypad = Keypad(makeKeymap(keys), pin_rows, pin_column, ROW_NUM, COLUMN_NUM);

void setup() {

lcd.begin(16, 2);

lcd.setCursor(0,0);

lcd.print("LCD is");

lcd.setCursor(0,1);

lcd.print("running");

}

void loop() {

char key = keypad.getKey();

if (key == '0') {

displayStudentInfo();

delay(1000);

}

}

void displayStudentInfo() {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("0 is");

lcd.setCursor(0, 1);

lcd.print("Pressed!");

}

PIR Sensor

Circuit Diagram:

Code:

#include <LiquidCrystal.h>

// const int rs = 12, en = 11, d4 = 5, d5 = 4, d6 = 3, d7 = 2;

LiquidCrystal lcd(12,11,5,4,3,2);

const int pirPin = 13;

void setup() {

lcd.begin(16, 2);

pinMode(pirPin, INPUT);

}

void loop() {

int motionStatus = digitalRead(pirPin);

lcd.clear();

lcd.setCursor(0, 0);

if (motionStatus == HIGH) {

lcd.print("Motion Detected");

} else {

lcd.print("Motionless");

}

delay(1000);

}

RGB LED (Increase Intensity)

const byte ledPin = 3;    // red RGB LED on Experimenter

const byte FADE_MAX = 255;  // max value for setting duty cycle

const byte FADE_INC = 5;  // increment for changing duty cycle

void setup()

{

pinMode(ledPin, OUTPUT);

}

void loop()

{

int fadeValue;  // PWM value

// fade in from min to max in increments of 5 points:

for(fadeValue = 0 ; fadeValue <= FADE_MAX; fadeValue +=FADE_INC)

{

analogWrite(ledPin, fadeValue); // sets the value (range from 0 to 255):

}

// fade out from max to min in increments of 5 points:

for(fadeValue = FADE_MAX; fadeValue >= 0; fadeValue -=FADE_INC)

{

analogWrite(ledPin, fadeValue); // sets the value (range from 0 to 255):

}

}

OR

int redPin= 6;

int greenPin = 5;

int bluePin = 4;

void setup() {

pinMode(redPin, OUTPUT);

pinMode(greenPin, OUTPUT);

pinMode(bluePin, OUTPUT);

}

void loop() {

setColor(255, 0, 0); // Red Color

delay(1000);

setColor(0, 255, 0); // Green Color

delay(1000);

setColor(0, 0, 255); // Blue Color

delay(1000);

setColor(255, 255, 255); // White Color

delay(1000);

setColor(170, 0, 255); // Purple Color delay(1000); } void setColor(int redValue, int greenValue, int blueValue) { analogWrite(redPin, redValue);

analogWrite(greenPin, greenValue);

analogWrite(bluePin, blueValue); }

CODE: Moisture Sensor

#include <LiquidCrystal.h>

const int sensorPin = A1;

LiquidCrystal lcd(13, 12, 5, 4, 3, 2);

int sensorValue;

double moisturePercentage;

void setup() {

lcd.begin(16, 2);

pinMode(sensorPin, INPUT);}

void loop() {

sensorValue = analogRead(sensorPin);

moisturePercentage = map(sensorValue, 0, 1023, 0, 100);

lcd.clear();

lcd.print("Soil Moisture: ");

lcd.print("%");

delay(500);}

OUTPUT:

Servo Motor

#include <Servo.h>

Servo servo1;  // Create object for Servo motor 1

Servo servo2;  // Create object for Servo motor 2

int position = 0;     // Variable to store the position

void setup()

{

servo1.attach(3);   // Set PWM pin 3 for Servo motor 1

servo2.attach(5);   // Set PWM pin 5 for Servo motor 2

}

void loop()

{

// Rotating Servo motor 1 in Anti clockwise from 0 degree to 180 degree

for (position = 0; position <= 180; position++)

{

servo1.write(position);  // Set position of Servo motor 1

delay(10);

}

// Rotating Servo motor 1 in clockwise from 180 degree to 0 degree

for (position = 180; position >= 0; position--)

{

servo1.write(position);  // Set position of Servo motor 1

delay(15);               // Short delay to control the speed

}

// Rotating Servo motor 2 in clockwise from 0 degree to 180 degree

for (position = 0; position <= 180; position++)

{

servo2.write(position);  // Set position of Servo motor 2

delay(10);               // Short delay to control the speed

}

// Rotating Servo motor 2 in Anti clockwise from 180 degree to 0 degree

for (position = 180; position >= 0; position--)

{

servo2.write(position);  // Set position of Servo motor 2

delay(15);               // Short delay to control the speed

} }

Simulation

Code for one DC Motor

int in1 = 10;

int in2 = 9;

int en = 8;

void setup() {

pinMode(in1, OUTPUT);

pinMode(in2, OUTPUT);

pinMode(en, OUTPUT);

}

void loop() {

digitalWrite(en, HIGH);

digitalWrite(in1, HIGH);

digitalWrite(in2, LOW);

delay(5000);

digitalWrite(en, LOW);

delay(1000);

digitalWrite(en, HIGH);

digitalWrite(in2, HIGH);

digitalWrite(in1, LOW);

delay(5000);

digitalWrite(en, LOW);

delay(1000);

}

Code 2 DC motor

// Define motor control pins for left motor

int leftMotorIn1 = 10;

int leftMotorIn2 = 9;

int leftMotorEn = 8;

// Define motor control pins for right motor

int rightMotorIn1 = 6;

int rightMotorIn2 = 5;

int rightMotorEn = 7;

// Define button pins

int forwardButton = 1;

int reverseButton = 2;

int leftButton = 3;

int rightButton = 4;

void setup() {

// Set up left motor pins

pinMode(leftMotorIn1, OUTPUT);

pinMode(leftMotorIn2, OUTPUT);

pinMode(leftMotorEn, OUTPUT);

// Set up right motor pins

pinMode(rightMotorIn1, OUTPUT);

pinMode(rightMotorIn2, OUTPUT);

pinMode(rightMotorEn, OUTPUT);

// Set up button pins

pinMode(forwardButton, INPUT_PULLUP);

pinMode(reverseButton, INPUT_PULLUP);

pinMode(leftButton, INPUT_PULLUP);

pinMode(rightButton, INPUT_PULLUP);

}

void moveForward() {

digitalWrite(leftMotorEn, HIGH);

digitalWrite(leftMotorIn1, HIGH);

digitalWrite(leftMotorIn2, LOW);

digitalWrite(rightMotorEn, HIGH);

digitalWrite(rightMotorIn1, HIGH);

digitalWrite(rightMotorIn2, LOW);

}

void moveReverse() {

digitalWrite(leftMotorEn, HIGH);

digitalWrite(leftMotorIn1, LOW);

digitalWrite(leftMotorIn2, HIGH);

digitalWrite(rightMotorEn, HIGH);

digitalWrite(rightMotorIn1, LOW);

digitalWrite(rightMotorIn2, HIGH);

}

void turnLeft() {

digitalWrite(leftMotorEn, HIGH);

digitalWrite(leftMotorIn1, LOW);

digitalWrite(leftMotorIn2, HIGH);

digitalWrite(rightMotorEn, HIGH);

digitalWrite(rightMotorIn1, HIGH);

digitalWrite(rightMotorIn2, LOW);

}

void turnRight() {

digitalWrite(leftMotorEn, HIGH);

digitalWrite(leftMotorIn1, HIGH);

digitalWrite(leftMotorIn2, LOW);

digitalWrite(rightMotorEn, HIGH);

digitalWrite(rightMotorIn1, LOW);

digitalWrite(rightMotorIn2, HIGH);

}

void stopMotors() {

digitalWrite(leftMotorEn, LOW);

digitalWrite(rightMotorEn, LOW);

}

void loop() {

// Check the status of each button

if (digitalRead(forwardButton) == LOW) {

moveForward();

} else if (digitalRead(reverseButton) == LOW) {

moveReverse();

} else if (digitalRead(leftButton) == LOW) {

turnLeft();

} else if (digitalRead(rightButton) == LOW) {

turnRight();

} else {

stopMotors();

}

}

LM35

int tempPin=A0;

float temp;

#include <LiquidCrystal.h>

LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

void setup()

{

Icd.begin (16, 2);

lcd.print("Temperature=");

}

void loop()

{

temp = analogRead (tempPin);

temp = temp*0.48828125;

delay (1000);

Icd.setCursor (0, 1);

lcd.print(temp);

lcd.print(" Centigrade");

}

Simulation screenshot :

To Display Light intensity in Percentage(LDR)

#include <LiquidCrystal.h>

// Initialize the LCD with the specified pins

LiquidCrystal lcd(13, 12, 5, 4, 3, 2);

const int ldrPin = A0; // LDR connected to analog pin A0

void setup() {

lcd.begin(16, 2);  // Initialize the LCD with 16 columns and 2 rows

lcd.print("Light Intensity:");

pinMode(ldrPin, INPUT);

}

void loop() {

int ldrValue = analogRead(ldrPin); // Read analog value from the LDR

float intensity = map(ldrValue, 0, 1023, 0, 100); // Map the value to percentage (0-100%)

lcd.setCursor(0, 1);

lcd.print(intensity);

lcd.print("%"); // Display light intensity percentage

delay(1000); // Wait for a second

}